| # HELIUM ILLUSTRATED REPORT |  |  |  |  | Brilliant 0.2309 ct |  |  |  |  |  |  |  |  | Brilliant 0.2420 ct |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rough diamond |  |  | July 21, 2010 |  | Clarity: VVS2, Color: H, Grade: 0 |  |  |  |  |  |  |  |  | Clarity: VVS2, Color: H, Grade: 0 |  |  |  |  |  |  |  |
|  |  |  |  |  | Rough weight | 0.4524, ct |  |  |  | Yield | 51.11, % |  |  | Rough weight |  | 0.5362, ct |  |  | Yield | 45.28, % |  |
|  |  |  |  |  | Tilting | 66.71, ° |  |  |  | Next part | 0.5924, ct |  |  | Tilting |  | 112.92, ° |  |  | Next part | 0.5777, ct |  |
| Rough weight, ct |  | 1.0448 (0.5362 + 0.4524) |  |  | Distance to Holder | 3.755, mm |  |  |  | Price | 414.00, $ |  |  | Distance to Holder |  | 3.756, mm |  |  | Price | 432.00, $ |  |
| Diamonds weight, ct |  | 0.4729 (0.2309 + 0.2420) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Yield, % |  | 45.19 (45.28, 51.11) |  |  | Parameters |  |  |  |  |  | Grade |  |  | Parameters |  |  |  |  |  | Grade |  |
| Saw plane width, mm |  | 0.180 |  |  | Diameter min, max |  | 3.962, mm |  |  | 3.942, mm | 0 |  |  | Diameter min, max | 4.034, mm |  |  |  | 4.054, mm | 0 |  |
| Saw quantity |  | 1 |  |  | Girdle ratio |  | 0.995 |  |  |  |  |  |  | Girdle ratio | 1.005 |  |  |  |  |  |  |
| Saw weight |  | 0.0562, ct; 5.38 % |  |  | Pavilion angle |  | 40.75, ° |  |  |  | N/A |  |  | Pavilion angle | 41.20, ° |  |  |  |  | N/A |  |
| Appraiser |  | AGS_Classic |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Price list |  | Rapaport: 09 july 2004 |  |  | Crown angle |  | 35.00, ° |  |  |  | 0 |  |  | Crown angle | 35.10, ° |  |  |  |  | 0 |  |
| Diamonds price, $ |  | 846.00 (414.00 + 432.00) |  |  | Total depth |  | 61.17, % |  |  | 2.423, mm | N/A |  |  | Total depth | 61.18, % |  |  |  | 2.468, mm | N/A |  |
|  |  |  |  |  | Pav depth bezel |  | 43.08, % |  |  | 1.707, mm | 0 |  |  | Pav depth bezel | 43.77, % |  |  |  | 1.766, mm | 0 |  |
|  |  |  |  |  | Crown height bezel |  | 14.88, % |  |  | 0.589, mm | N/A |  |  | Crown height bezel | 14.93, % |  |  |  | 0.602, mm | N/A |  |
|  |  |  |  |  | Girdle bezel |  | 3.20, % |  |  | 0.127, mm | N/A |  |  | Girdle bezel | 2.47, % |  |  |  | 0.100, mm | N/A |  |
|  |  |  |  |  | Pav depth valley |  | 43.94, % |  |  | 1.741, mm | N/A |  |  | Pav depth valley | 44.64, % |  |  |  | 1.801, mm | N/A |  |
|  |  |  |  |  | Crown height valley |  | 15.72, % |  |  | 0.623, mm | N/A |  |  | Crown height valley | 15.78, % |  |  |  | 0.637, mm | N/A |  |
|  |  |  |  |  | Girdle valley |  | 1.50, % |  |  | 0.059, mm | 0 |  |  | Girdle valley | 0.75, % |  |  |  | 0.030, mm | 0 |  |
|  |  |  |  |  | Table |  | 57.50, % |  |  | 2.278, mm | 0 |  |  | Table | 57.50, % |  |  |  | 2.319, mm | 0 |  |
|  |  |  |  |  | Culet |  | N/A, % |  |  | N/A, mm | N/A |  |  | Culet | N/A, % |  |  |  | N/A, mm | N/A |  |
|  |  |  |  |  | Lower facets |  | 82.00, % |  |  |  | N/A |  |  | Lower facets | 82.00, % |  |  |  |  | N/A |  |
|  |  |  |  |  | Culet through bezel |  | 10.58, ° |  |  |  | 0 |  |  | Culet through bezel | 10.41, ° |  |  |  |  | 0 |  |
|  |  |  |  |  | Fish eye effect |  | 49.88, ° |  |  |  | 0 |  |  | Fish eye effect | 48.10, ° |  |  |  |  | 0 |  |
| Notes: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |